VT-31 Flight Briefing Guide

(Updated Sep 2018)

Communications:

1. Radio Procedures and Identification:  “Our identification will be (Montana XX).  The PM will handle all comms, and the PF will monitor and read back clearances when received.”

2. Frequencies: “We will be using the 20 preset UHF frequencies as well as monitoring Montana base and backing ourselves up with the VHF radio which is primary at all civilian airfields.”

3. CRM: “Crew coordination will be in accordance with NATOPS including two challenge rule, sterile cockpit, and mandatory callouts. A training time-out requires a verbal request only. “

Weather:

1. Local Observation: Review local METAR and any PIREPS

2. Enroute and Destination Forecast: Review DD175-1

3. Alternate Forecast: Identify appropriate alternate given forecasted weather.

Flight Planning:

1. Departure: “We will plan to depart via course rules/IFR flight plan.”

2. Mission / Fuel Planning: “We will be flying (C410l, 14202, etc) mission(s) and will plan the training accordingly. We will consider 265 lbs per side “minimum fuel” and 200 lbs per side “emergency fuel”. A fuel packet (will/will not) be required for today’s profile.”

3. Recovery: “We are planning a (course rules/ instrument approach) recovery at our planned destination.”

Emergencies:

1. Aborting Takeoff: “If anyone sees the need to abort, call out ‘abort, abort, abort’, the pilot at the controls will bring the power levers to idle, utilize reverse and brakes as required and discuss/troubleshoot the malfunction once clear of the runway.”

2. Divert Fields: Brief viable divert options based on planned profile.

3. Radio Failure: “In the event of a radio failure, we will troubleshoot for stuck mic and try other frequencies including Guard. Lost comm recovery procedures will be in accordance with the local Letter of Agreement and FAR/AIM as appropriate.”

4. Downed Pilot: “If we are the first on-scene, the student will take the controls and assume an orbit around the crash site, while the IP runs the on-scene commander checklist. We will remain on station until a logical conclusion.”

5. System Failures: “We (are/are not) planning simulated system failures today. The IP will preface any simulated failure by stating ‘SIMULATED’. If we have an actual emergency during a simulated emergency, the IP will clean up the simulated scenario and the actual emergency will be handled as a crew.”

6. Spin/Windshear/Forced Landing/Ditch: “In the event of a spin, windshear condition, forced landing, or ditch, the IP will take controls and execute NATOPS directed procedures.” The pilot not at controls will back up the control inputs to ensure proper execution.”

“The memory items for a spin are…”

“The memory items for windshear are…”

7. Emergency Egress: “The observer will be the first member to exit the aircraft. The air stair door is considered the primary exit. Utilization of the emergency exit hatch is at the discretion of the IP. If required, the observer will remove the liferaft and deploy it in accordance with NATOPS outside of the aircraft. The second student is responsible for the first aid kit. The IP will be the last out of the aircraft.”

Observer Duties:

“The observer will…

1. Scan for Traffic

2. Confirm gear down and locked

3. Monitor radios

4. Count landings

5. Transport of any passengers will be in accordance with TW-4 SOP’s. A safety brief will be provided by the aircraft commander as required.”

Stan Board Minutes / Read & Initial:

- Ensure all crew members are up to date.