| AIR REFUELING KNEEBOARD |  |  |  |  |  |  |  | Timeline |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Flight Call Sign: |  |  |  |  |  |  |  | Brief | -2+15 |  |  |
|  | Side # |  | Buno # |  |  | Parking |  | Walk | -0+50 |  |  |
| Lead |  |  |  |  |  |  |  | Start | -0+30 |  |  |
| Wing |  |  |  |  |  |  |  | Check-In | -0+25 |  |  |
| COMM PLAN |  |  |  |  |  |  |  | Taxi | -0+15 |  |  |
| Primary |  | Secondary |  |  | A/A Tacan |  |  | Takeoff |  |  |  |
| 140.525 |  | 140.625 |  |  | 29Y / 92Y |  |  | Land |  |  |  |
| Forecast WX: |  |  |  |  |  |  |  |  |  |  |  |
| Notes: |  |  |  |  |  |  |  |  |  |  |  |
| Rendezvous/Pre-descent Checklist 1. Radios ………...…..…..…...……..Set (P,CP) 2. Crew..……………..……....….Notified (P,CP) 3. Radar (receiver)..……..……....Standby (CP) 4. TAS…………….........................As Req (CP) 5. Altimeters….…………………As Req (P,CP) 6. TACAN A/A……..……….…......As Req (CP) 7. MARSA (Tanker) …………….....Declare (P) Prep for Contact (Completed by 1/2 mile) 1. Crew………..…..…..Ready for Refueling (P) 2. Exterior Lights……..…..…...…..As Req (CP) 3. Autopilot…………...................As req (P,CP) 4. Transponder (receiver)……....….....Off (CP) 5. Seat Height...….……...Adjust as req (P,CP) Simulated Contact 1. Position…….....…….….Monitor (P,CP, OB) After Air Refueling 1. After Refueling Report……..….As Req (CP) 2. External Lights …..………….....As Req (CP) 3. Transponder ……..………...……….Set (CP) 4. TAS……………………..……… As Req (CP) 5. Altimeters………..……...………..Set (P, CP) 6. Crew …………………..………….Notified (P) |  |  |  | ARCP ARIP |  |  | Contingencies |  |  |  |  |
|  |  |  |  |  |  |  | MX Delay |  |  |  |  |
|  |  |  |  |  |  |  | Abort |  |  |  |  |
|  |  |  |  |  |  |  | Breakaway (prac) |  |  |  |  |
|  |  |  |  |  |  |  | Formation EP’s |  |  |  |  |
|  |  |  |  |  |  |  | Speed Schedule |  |  |  |  |
|  |  |  |  |  |  |  | 3 nm |  |  | 180 |  |
|  |  |  |  |  |  |  | 2 nm |  |  | 180 |  |
|  |  |  |  |  |  |  | 1 nm |  |  | 180 |  |
|  |  |  |  |  |  |  | 1/2 nm |  |  | 170 |  |
|  |  |  |  |  |  |  | 0.3 nm |  |  | 160 |  |
|  |  |  |  |  |  |  | Pre-contact |  |  | 150 |  |

|  |  | Roll Out Distance in NM |  |  |  | 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25 degrees AOB |  | Tanker TAS |  |  |  | 159 |  |  |
| Tanker IAS | 150 | Receiver TAS |  |  |  | 188 |  |  |
| Receiver IAS | 180 |  |  |  |  |  |  |  |
| Closure |  | Drift |  |  |  |  |  |  |
| 347 |  | 15L | 10L | 5L | 0 | 5R | 10R | 15R |
| Offset |  | 0.7 | .9 | 1.1 | 1.3 | 1.5 | 1.8 | 2.0 |
| Turn Range |  | 4.7 | 4.9 | 5.1 | 5.3 | 5.4 | 5.6 | 5.8 |

Air Refueling Altitude 5K

|  |  | Roll Out Distance in NM |  |  |  | 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25 degrees AOB |  | Tanker TAS |  |  |  | 170 |  |  |
| Tanker IAS | 150 | Receiver TAS |  |  |  | 200 |  |  |
| Receiver IAS | 180 |  |  |  |  |  |  |  |
| Closure |  | Drift |  |  |  |  |  |  |
| 370 |  | 15L | 10L | 5L | 0 | 5R | 10R | 15R |
| Offset |  | .8 | 1.0 | 2 | 1.5 | 1.7 | 2.0 | 2.3 |
| Turn Range |  | 5.0 | 5.2 | 5.4 | 5.7 | 5.9 | 6.0 | 6.2 |

Air Refueling Altitude 8K

|  |  | Roll Out Distance in NM |  |  |  | 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25 degrees AOB |  | Tanker TAS |  |  |  | 178 |  |  |
| Tanker IAS | 150 | Receiver TAS |  |  |  | 210 |  |  |
| Receiver IAS | 180 |  |  |  |  |  |  |  |
| Closure |  | Drift |  |  |  |  |  |  |
| 388 |  | 15L | 10L | 5L | 0 | 5R | 10R | 15R |
| Offset |  | 0.9 | 1.1 | 1.3 | 1.6 | 1.9 | 2.2 | 2.5 |
| Turn Range |  | 5.2 | 5.5 | 5.7 | 6.0 | 6.2 | 6.4 | 6.6 |

Figure 6-19, Air Refueling Altitude 11K