T-44C Operational Limits and Memory Items

| ## Airspeeds (KIAS) | ### Memory Items |  |
| --- | --- | --- |
| VMO/VNE Max dive/level flight 227 | ##### Emergency Shutdown On Deck | Emergency Shutdown Checklist |
| # VMCA Minimum controllable airspeed 86 | ##### 1) Condition Levers – FUEL CUTOFF | 1) Power lever – IDLE |
| VX Best angle of climb 102 | If confirmed/suspected fire or fuel leak, continue checklist. | 2) Prop lever – FEATHER |
| VY Best rate of climb 108 | Otherwise, secure using Secure Checklist. | 3) Condition lever – FUEL CUTOFF |
| VSSE Minimum safe 1 engine inop 91 | 2) Firewall Valves – CLOSED | If prop fails to feather, alternate feathering checklist. |
| VXSE Best angle of climb single engine 102 | 3) Boost Pumps – OFF | If confirmed/suspected fire or fuel leak, continue steps 4-6. |
| VYSE Best rate of climb single engine 110 | 4) Fire Extinguisher – As Required | 4) Firewall valve – CLOSED |
| VFE Max flap extension/extended (full) 140 | 5) AUX BATT Switch – OFF | 5) Fire Extinguisher – As Required |
| VFE Max flap extension/extended (appr) 174 | 6) Gang Bar – OFF | 6) Bleed air – CLOSED |
| VLR Max landing gear retraction 145 | 7) Evacuate Aircraft |  |
| VLE Max landing gear extension 155 |  | Single-Engine Waveoff/Missed Approach |
| VA Maneuvering speed 153 | ### Abnormal Start | 1) Power – Max Allowable, establish positive rate of climb |
| VMCG Min controllable speed on ground 63 | ### 1) Condition Lever – FUEL CUTOFF (dec. below 790C) | (VXSE minimum) |
| Max range glide 130 | 2) Starter – Starter only (for the remainder of the 40 seconds) | 2) Flaps – APPROACH (unless already UP) |
| Max endurance glide 102 | 3) Starter – OFF (at 40 seconds) | 3) Gear - UP |
|  |  | 4) Flaps - UP |
| ## Electrical | Aborting Takeoff | 5) Prop – 1900 RPM |
| DC generator voltage 28.25  .8 VDC | ### 1) Announce “Abort” |  |
| Min battery voltage for APU charge 18 VDC | 2) Power levers – IDLE | ### Jammed Power Lever |
| Min battery voltage for APU start 20 VDC | 3) Reverse – As Required | 1) Condition Lever – FUEL CUTOFF |
| Min battery voltage for BATT start 22 VDC | 4) Brakes – As Required | 2) Emergency Shutdown Checklist – Execute |
| Starter (40sec, 60sec) x2, then 40 on, 30min off | 5) Condition levers – FUEL CUTOFF |  |
|  | 6) Firewall valves – CLOSED | Fuel Leak |
| Interstage Turbine Temperature (ITT) | 7) Boost pumps – OFF | 1) Condition Lever – Fuel Cutoff |
| Normal operating range 400–790 C | 8) Fire extinguisher(s) – As Required | ##### 2) Emergency Shutdown Checklist – Execute |
| Max allowable 790 C | 9) AUX BATT switch – OFF |  |
| Max continuous 790 C | 10) Gang bar – OFF | Primary Governor Failure/Malfunction |
| Max reverse 790 C | 11) Evacuate aircraft | 1) Attempt to adjust prop rpm to normal operating range. |
| Max acceleration (up to 2 sec) 850 C |  | ### 2) Power Lever – IDLE |
| Max low idle 685 C | Engine Failure After Takeoff | 3) Prop Lever – FEATHER |
| Max cruising 740 C | 1) Power – As Required | 4) Alternate Prop Feathering Checklist – As Required |
| Cruise climb 765 C | 2) Gear – UP |  |
| Start Limit 925 C (cutoff ) 1090 C (2 sec) | 3) Airspeed – As Required (VXSE or VYSE) | Generator Failure |
|  | 4) Emergency Shutdown Checklist – Execute | 1) Starter – OFF |
| Torque |  | ### 2) Generator – OFF, Reset Momentarily, release slowly to ON |
| Normal operating range 400–1315 ft-lb | ##### EGPWS Pull Up Warning (IMC or at Night) | If generator will not reset: |
| Max @ 1900 RPM 1520 ft-lb | 1) Wings – Level | 3) Generator – OFF |
| Max @ 2200 RPM 1315 ft-lb | #### 2) Power – Max Allowable | 4) Current limiter (Battery Ammeter) – Checked |
| Max acceleration 2100 ft-lb | 3) Pitch – As required to set and maintain VX |  |
|  | 4) Flaps – Approach (unless already up) | Smoke and Fume Elimination |
| ### Turbine Tachometer (N1) | ##### 5) Gear – Up | 1) Oxygen masks/MIC switches (100 percent) – As Required |
| Normal Operating Range 51-101.5 % | 6) Flaps – Up | ### 2) Pressurization – DUMP |
| Low idle range 51-54 % | 7) Props – 1900 rpm |  |
| Hi idle range 70-73 % | 8) Continue climb at VX until all visual and voice warnings cease | Explosive Decompression |
| Max reverse 86 % |  | 1) Oxygen masks/MIC switches (100 percent) – As Required |
| Max acceleration 102.6 % | Low Altitude Windshear | ### 2) Descend – As Required |
| Max continuous 101.5 % | 1) Power – Maximum Allowable |  |
|  | 2) Attitude – Set and Hold Approximately 15° Noseup | Smoke/Fire of Unknown Origin |
| Propeller Tachometer (N2) | 3) Gear – UP | 1) Crew – Alerted |
| Normal operating range 1800-2200 rpm | 4) Flaps – Maintain Current Setting | 2) Cabin temperature mode – OFF |
| Max reverse 2100 rpm |  | 3) Vent blower – AUTO |
| Max during accel (2sec) 2420 rpm | Spin/Out of Control Flight Recovery | 4) Oxygen masks/MIC switches (100 percent) – As Required |
|  | 1) Power Levers – Idle |  |
| Oil Temperature | 2) Rudder – Full deflection opposite direction of turn needle | Emergency Descent Procedure |
| Normal operating range 10-99 C | 3) Control Wheel – Rapidly forward | 1) Power Levers - IDLE |
| Max oil temp 99 C | 4) Rudder – Neutralize after rotation stops | 2) Props – Full Forward |
|  | 5) Control wheel – Pull out of dive by exerting smooth, | 3) Flaps – As Required |
| Oil Pressure | steady back pressure. | 4) Landing Gear – As Required |
| Normal operating range 85-100 psi |  | 5) Airspeed – As Required |
| ### Max oil pressure 100 psi | Windmilling Airstart | 6) Windshield Heat – As Required |
| Min oil pressure 40 psi | 1) Power Lever (Failed Engine) – IDLE |  |
|  | 2) Prop Lever (Failed Engine) – Full Forward | General |
| Weights | 3) Condition Lever (Failed Engine) – FUEL CUTOFF | Pneumatic pressure norm operating range 12-20 psi |
| ### Max ramp weight 9710 lbs | 4) Firewall Valve – OPEN | Max sink rate on landing 600 fpm |
| Max takeoff weight 9650 lbs | 5) Autoignition – ARMED | Max crosswind 20 knots |
| Max landing weight 9168 lbs | ### 6) Condition Lever – LOW IDLE | Max cabin pressure differential 4.7 psi |
|  | 7) Power – As Required | Gyro suction norm operating range 4.3-5.9 inHg |
|  |  | Min oxygen required for local/XC flight 1000/1500 psi |
|  |  | Prop deicer ammeter normal operation 14-18 amps |
|  |  | Normal TAS altitude range +/- 2700 feet |
|  |  | Updated 9 April 2019 |